### Methodology and Working Logic

- IS 875–compliant wind and sag calculations are carried out for transmission line towers using tower coordinates, span data, and conservative design assumptions.

- Historical weather data is fetched only for a limited number of geographically representative locations using a grid-based sampling approach; remaining towers are mapped to the nearest sample to minimize API usage.

- Minimum temperature, maximum temperature, and 95th-percentile wind speed are obtained from a public reanalysis dataset and stored using a persistent cache to ensure repeatability.

- The governing design wind speed is conservatively taken as the higher of the IS 875 basic wind speed and the derived weather wind speed.

- Wind pressure is calculated using the IS 875 relation ( p = 0.6 V^2 ), and resultant conductor load is computed by combining wind and vertical loads.

- Sag under hot and wind conditions is evaluated using standard parabolic sag equations with fixed conductor properties, span length, and design tension.

- Fail-safe features such as rate-limit protection, progress saving, fallback values, and caching ensure reliable and uninterrupted execution.

- Final tower-wise results are consolidated into a single Excel output for preliminary design review.

### Explanation for Constant Values

- Several parameters remain constant as they are code-defined inputs prescribed by IS 875 (Part 3) and do not vary with individual coordinates.

- Towers within the same wind zone share identical basic wind speed (Vb) as per the IS 875 wind zoning map.

- Risk coefficient (k₁), terrain/height factor (k₂), and topography factor (k₃ = 1.0) are kept uniform due to consistent assumptions regarding structure category, terrain, reference height, and absence of significant topographical features.

- Consequently, wind pressure values remain uniform for locations governed by the same design wind speed.

### Justification and Scope for Improvement

- The approach is intentionally conservative and fully compliant with IS 875, making it suitable for safe preliminary design in the absence of detailed site-specific inputs.

- The model can be refined at the detailed design stage by introducing height-wise k₂ variation, GIS-based wind zoning, topography-dependent k₃ evaluation, and structure-specific importance factors.